Q7 HD Transmitter

Overview

Q7 HD transmitter is a specially developed new high-definition transmission equipment to solve large-area displays, has strong signal reception ability of video. DVI port can receive 2560 * 1536 HD graphics signal, while the four Ethernet ports output, Q7 can be used to support the large-area displays which maximum width reaches 2560 pixels, and the maximum height reaches 1536 pixels. Just one high-resolution graphics can easily achieve a large-area displays control.

Meanwhile, Q7 also has rich, useful features, such as the time lock screen function, playing the default image on display when there is no signal, on-board test function, on-board temperature detection function.

Q7 HD transmitter is the most powerful synchronous transmission equipment with the strongest reception and the largest load area currently on the market.

Features

• Top effects, maximum input resolution support up to 2560 * 1536 pixels, and automatic match graphics resolution.

• Large control area: four Ethernet output ports, one Q7 card control area: maximum width reaches 2560 pixels, and the maximum height reaches 1536 pixels.)

• Full screen test function to facilitate the commissioning and aging test.

• Button to adjust brightness for 16 levels, immediate effect, and automatically save to send cards;

• Real-time parameter: the display parameters can be save in and be send from sender cards, to facilitate the site cabinet installation;

• Time management function, to follow up and manage the project process.

• Default image display function, show the customer default picture (eg: logo) in the absence of the signal source.

• Support all the 5A series receiving cards or Colorlight.

Applicable occasions

Performing stages, large-display and other occasions.

Pecifications

| Video interface |  |
| --- | --- |
| Video Interface | DVI or HDMI , Connect to a computer graphics or video processor |
| Input resolution | Maximum supported resolution2560*1536,supports 1920*1080,1280*1024 |
| Adaptive Resolution | Can be set to any resolution, so that graphics output and LED display can be exactly consistent |
| Video source frame rate | 60Hz |
| Gigabit Ethernet port output |  |
| Output port | 4*1Gbps，supports 4 Gigabit Ethernet ports output. |
| Control area | Single network port to control an area of 655360 pixels, 4 Ethernet ports, compared with 2621440 pixels (maximum width reaches 2560 pixels, and the maximum height reaches 1536 pixels.) |
| Transmission distance | UTP cable≤140M CAT6 cable≤170M OPTIC FIBER transmission distance unrestricted |
| Port splicing | 4 Ethernet ports can be spliced ​​in any way ,such as up and down or left and right. |
| Data transfer format | In accordance with the gigabit network frame format. And append CRC. |
| Device compatibility |  |
| Matching equipment | 5A/5A-S/5A-F/5A-Pro/5A-Mini |
| Peripheral equipment | Multifunction card, fiber optic transceivers, Gigabit switches. |
| Specifications |  |
| Size | 1U Standard chassis |
| Input Voltage | AC 100~260V |
| Power | 5W |
| Weight | 200g |
| Memory width | 128bit |
| Human-machine interface |  |
| Configuration parameters interface | USB, RS233 serial port |
| DVI Information Display | To show frame rate, blanking and clock information. |
| Button to adjust brightness | Support, 16 levels brightness to adjust and work in real time and automatically store in the sending card. |
| Real-time adjustment parameters | Gamma, the control area, control parameters. |
| Brightness & Color Temperature Adjustment | Supports |
| Intelligent Detection Systems | Supports |
| Real-time clock | Real-time clock with the back-up battery, for the working time statistic. |
| Accessibility |  |
| Cascading multiple cards | Support. |
| Multi-screen settings | Support the same size and different sizes of multiple displays |
| Background play | Support (extended mode) |
| Audio transmission | Supports |
| Network error probability of detection | Supports |

⑦

⑥

Hardware

⑤

④

③

②

①

1、Interface

| S/N | Name | Function | Remarks |
| --- | --- | --- | --- |
| 1 | Power switch | Power switching devices |  |
| 2 | Power Input | power supply interface( AC 100~260V) |  |
| 3 | USB | USB debugging interface |  |
| 4 | DVI | DVI signal input port |  |
| 5 | Network port | RJ45, 4-output network port, connect to the receiver card |  |
| 6 | LCD Screen | Show brightness levels and test mode |  |
| 7 | Button | Adjust brightness and test mode |  |

2、Dimensions (1U standard chassis)