T7 Sender Card

Overview

T7 sender card, compared with other sender cards on the market Currently, adopts a better power solution, and has better reliability. Two network outputs can be set random area of the scream, and control different LED display.

T7 sender card is seamlessly compatible with Gigabit Ethernet Transmission System, one of Colorlight pioneered technology. It is nice not only to meet requirements of external signals, but also to further meet the diverse needs of customers.

Features

- Full-color sending card, support full-color, dual-color, monochrome display

- Single card supports a maximum resolution of 2048 * 1365 pix

- Single card supports a maximum size of 1280 * 1024 points, Width up to 2048 points, and height up to 1365 points

- Support for audio input

- USB interface control

- Support for standard network transmission equipment, through gigabit fiber optic transceivers, Gigabit switches increase the transmission distance

- Wide voltage design, more adaptable, more stable performance

Specifications

| Video interface |  |
| --- | --- |
| Video Interface | DVI or HDMI , connect to a computer graphics or video processor |
| Input resolution | Selectable control area (1024*768、1280*720、1280*1024、1600*768、1920*640) |
| Adaptive Resolution | Can be set to any resolution, so that graphics output and LED display can be exactly consistent |
| Video source frame rate | 60Hz |
| Gigabit Ethernet port output |  |
| Output port | 2*1Gbps，supports two Gigabit Ethernet ports output. |
| Control area | Single network port to control an area of 655360 pixels, dual Ethernet ports, compared with 1310720 pixels (maximum width reaches 2048 pixels, and the maximum height reaches 1365 pixels.) |
| Transmission distance | UTP cable≤140M CAT6 cable≤170M OPTIC FIBER transmission distance unrestricted |
| Port splicing | 2 Ethernet ports can be spliced ​​in any way ,such as up and down or left and right. |
| Data transfer format | In accordance with the gigabit network frame format. And append CRC. |
| Device compatibility |  |
| Matching equipment | 5A/5A-S/5A-F/5A-Pro/5A-Mini |
| Peripheral equipment | Multifunction card, fiber optic transceivers, Gigabit switches. |
| Specifications |  |
| Size | 136mm*100mm |
| Input Voltage | 3.3V-12V wide voltage |
| Power | 3W |
| Weight | 100g |
| Human-machine interface |  |
| Configuration parameters interface | USB, RS233 serial port |
| Brightness & Color Temperature Adjustment | Supports |
| Intelligent Detection Systems | Supports |
| Accessibility |  |
| Cascading multiple cards | Support. |
| Multi-screen settings | Support the same size and different sizes of multiple displays |
| Background play | Support (extended mode) |
| Audio transmission | Supports |
| Network error probability of detection | Supports |

Hardware

⑥

⑦

⑧

④

⑤

①

③

②

1、Interface

| S/N | Name | Function | Remarks |
| --- | --- | --- | --- |
| 1 | Indicate lamp | Indicate power and signal transmission status | red for power, green for signal |
| 2 | Network port A | RJ45，For transmitting data signals | The dual network ports can achieve import/export at random |
| 3 | Network port B | RJ45，For transmitting data signals |  |
| 4 | USB | Debug Interface |  |
| 5 | DVI | Connect to the computer graphics DVI signals |  |
| 6 | Power 1 | Connect DC5V power supply for the receiver card | Only one is used. |
| 7 | Power 2 | Connect DC5V power supply for the receiver card |  |
| 8 | PCI slots | PCI slot into the computer, to send the card supply |  |

2、Indicator Light functions

Red: ON for power available

Green: ON/OFF quick flash (about 5-10 times/second) indicates that the data signal transmission is normal.

3、Figure for receiving card size and hole position

Unit: mm